# Forecasting Marine and Ship-Mooring Parameters: Analytical Summary

Executive Summary: We surveyed forecasting methods for each listed parameter. Environmental variables (currents, waves, tides, water quality, wind, etc.) are forecast using a mix of physics-based and data-driven approaches. Short-range horizons (hours to days) typically rely on high-resolution models and machine learning (ML); longer horizons use numerical models or statistical trends. Data availability and resolution vary: e.g. tides require years of observations for harmonic analysis, waves can be forecast hourly from wind, while currents often use ocean models with data assimilation. Below we provide a parameter-by-parameter forecast outlook: techniques, horizons, resolutions, data needs, features, uncertainty quantification, and key references.

| Parameter | Unit | Techniques (Stat/ML/DL) | Forecast Horizon (best) | Resolution & Enabling Method | Data Length & Quality | Inputs/Sensors | Uncertainty | Ref. (marine context) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compass | degrees | – (navigational sensor; not environment-driven) | N/A | – | – | – | – | Not forecastable (no physical process). |
| Current Speed | m/s | Physical models (hydrodynamic + data assimilation); Statistical ARIMA/SARIMA; Kalman filtering; ML (RF, SVR); DL (LSTM, CNN, TCN, Transformer) | Short (hours–1 day): ML/DL (LSTM/CNN)【14†L1255-L1264】;<br>Medium (3–5 days): hybrid (assimilated ocean models);<br>Long (10+ days): dynamical models/ensemble | Hourly to sub-hourly with ML on buoy data; regional models yield ~1 km grid (hydrodynamic). ML requires dense sampling. | Multi-year current records (buoy/ADCP) plus boundary forcings. Gaps hurt ML performance. | Historical current time series; tidal constituents; wind and pressure fields【12†L181-L190】; bathymetry | Ensembles (perturbed model runs); Bayesian DL or MC dropout | Cheng et al. (2024) develop CNN+Wavelet for tidal currents, achieving low error (MAE ~0.025 over 10-step)【14†L1259-L1268】. Physically, tidal harmonic models can also predict currents. |
| Current Direction | degrees | Same as speed (vector component); wind interpolation | Short (hours–days): similar to speed | Hourly or finer; directional forecasts often accompany speed in models | Same as speed | Same as speed | Same as speed | No separate studies; direction follows speed and forcing patterns. |
| Tide Level | m | Harmonic analysis; ARIMA/SARIMA; SVM; LSTM/GRU; hybrid DL (decomposition+LSTM)【9†L49-L58】; Physical surge models | Very short (sub-day): ARIMA/ANN【9†L54-L63】;<br>Short (1–3 days): LSTM hybrids【9†L12-L21】;<br>Long (weeks): Harmonic (deterministic cycles) | 15 min–hourly data, any buoy; harmonic yields exact times years ahead (no uncertainty) | Years–decades of tide gauge data. Accurate long record yields best results. | Historical tide series (water level); solar/lunar ephemerides | Prediction intervals (ARIMA, Gaussian process); ensembles of model residuals | Huang et al. (2024) use VMD+LSTM on 1-year tide data, achieving RMSE ~0.04 m【9†L13-L21】. Traditional tidal prediction via harmonic analysis remains gold for long range, but ML improves short-term accuracy【9†L51-L58】. |
| Tide Pressure | hPa | Same as tide level (pressure sensor data instead of level) | Same as tide level | Hourly; pressure gauges record tide fluctuation | Same as tide level data needs | Same as tide level | Same as tide level | Equivalent to water level forecasting (pressure units). |
| Water Pressure | hPa | Same as tide; includes atmospheric component subtraction | Same as tide level | Hourly; pressure logs at mooring | Same as above | Tide-level, atmospheric pressure | Same as tide level | Often used as surrogate for tide/water level. |
| Water Temperature | °C | Physics (thermal ocean models); ARIMA; Kalman filter; ML (RF, SVR); DL (LSTM/GRU, CNN, CNN–LSTM)【31†L547-L556】 | Short (hours–days): CNN/LSTM【31†L547-L556】;<br>Medium (3–5 days): physics-ML hybrid;<br>Long (10+ days): climate model persistence | Hourly to daily; ML yields hourly forecasts at sensor locations; global models ~km scale | >1 year of hourly buoy or satellite data. Seasonal trends important. Quality (gaps, stratification) matter.【31†L559-L564】 | Recent water temp history; air temp, radiation, currents; historical SST data【31†L551-L560】 | Ensembles (model variants); Bayesian NN; quantiles | Farhangi et al. (2023) report CNN/LSTM models achieving low-error SST forecasts using multi-year data【31†L547-L556】. Spatiotemporal CNN–LSTM have also shown success for short-to-mid-term SST【31†L610-L619】【31†L628-L637】. |
| Dew-Point Temp. | K | Meteorological NWP; Regression; ML (ANN, LSTM) | Short (hours): ML nowcasting;<br>Medium (1–3 days): NWP ensemble | Hourly or 10-min if sensor data; often reported per station | Months–years of meteorological station data | Air temp, RH (or virtual temp), pressure | Ensembles (NWP); prediction intervals | Standard forecast by weather models; rarely treated as independent target. |
| Precipitation (Diff) | mm | NWP; Persistence; ML (RF, Gradient Boost, CNN); Convective nowcasting (radar/LSTM) | Very short (minutes to hours): radar nowcasting;<br>Short (0–3 days): NWP ensembles | Minute-to-hourly (radar nowcast) to daily (forecast models) | Multi-year rain gauge/radar records. Highly variable; gaps critical. | Humidity, pressure, temperature; radar/satellite imagery | Ensembles; probabilistic (quantiles, Bayesian) | Subject of extensive weather research; no marine-specific studies. Over land and ocean, ensembles are standard (e.g. ECMWF ensemble for precipitation). |
| Precip. Intensity | mm/h | Same as precipitation | Same as precipitation | Same as precipitation | Same as precipitation | Same as precipitation | Same as precipitation | As above; intensity often derived from precip forecast. |
| Air Pressure | hPa | Meteorological (NWP, assimilation); ARIMA for local forecast | Short–medium (hours–3 days): local ARIMA; medium-long (up to 10 days): NWP | Hourly or finer from models; NWP global ~10 km, local ~1 km | Multi-decade reanalysis, surface obs (balloons/sat) | Historical pressure series; terrain data for local effects | NWP ensembles; variational data assimilation | Well-established atmospheric forecast; no maritime-specific approach needed. |
| Air Temperature | °C | NWP; ML (LSTM, CNN for nowcasts); Spatial interpolation | Short-term (hours): ML nowcasting; longer (days): NWP | Hourly; global models (~km), local sensor network | Decadal meteorological records; quality (homogenized) essential. | Historical temp; solar radiation; synoptic pattern | Same as pressure | As above; consider coupling with SST for marine boundary layers. |
| Relative Humidity | % | NWP; ML (RF, LSTM) | Same as air temperature | Hourly; sensor-level, often no spatial resolution (point) | Historical meteorological RH from stations (often derived) | Temperature, dew-point; vertical profiles | Same as air temperature | Forecast with atmospheric model; no special marine studies found. |
| Global Radiation | W/m² | Clear-sky models; NWP; ML (SVR, LSTM, CNN with satellite) | Short (minutes-hours): persistence/CNN;<br>Day-ahead: NWP solar radiation | 15-min to hourly; direct from radiation sensors or satellite-based models | Years of solar radiation records; influenced by clouds. | Historical radiation, cloud cover indices, satellite imagery | Ensemble weather model; prediction intervals | Mainly in solar energy research. E.g., satellite-driven ML models exist【42†L1-L4】. Less emphasis in marine forecasting. |
| Visibility (1-min avg) | (n/a) | NWP (fog models); ML classification/regression (ANN, SVM, XGBoost); hybrid (WRF+AI)【26†L625-L634】 | Nowcast to ~6h (fog formation) | Minutes; optical sensors or sky cameras. Models often hourly. | Years of visibility/meteorological observations at maritime sites. | Temperature, RH, wind, sea-spray; time-of-day | Probabilistic (fog probability); ensembles | AI and hybrid methods have been used for fog visibility in airport/marine context【26†L625-L634】; horizons ~1–6h. |
| Visibility (10-min avg) | (n/a) | Same as 1-min visibility | Same | Same as 1-min | Same as 1-min | Same as 1-min | Same as 1-min | – |
| Visibility (1-hr avg) | (n/a) | Same methods (often nowcast classification over longer window) | Hours (up to 12h with ML) | Hourly; can incorporate radar/lidar if available | As above | As above | As above | – |
| Visibility (24-hr avg) | (n/a) | NWP climatology or regression (often unnecessary; reflects last 24h) | N/A (used as metric, not forecasted) | Daily average; no forecast (resultant measure) | Dependent on many factors; rarely forecast as separate field. | N/A | N/A | Not typically forecast; used for statistics. |
| Wind Direction | degrees | NWP; ML (circular regression, LSTM); Data assimilation | Short (0–1 day): local ML; Medium (days): NWP ensemble | Hourly; model grid (10–100 km), or point forecast | Multi-year meteorological wind data | Geostrophic pressure, local terrain, historical wind | Ensembles; circular uncertainty | ML reviews show wind prediction focus on speed, but direction can be forecast similarly【22†L163-L170】. |
| Wind Speed | m/s | NWP; Kalman filter; ARIMA; ML (Random Forest, SVR, XGBoost); DL (LSTM, GRU, TCN, Transformer)【22†L154-L163】 | Very short (minutes): persistence/IR sensors;<br>Short (hours–days): LSTM/GRU【22†L154-L163】;<br>Medium (3–5 days): NWP ensemble;<br>Long (>7 days): global NWP climatology | 10 min to hourly; high-res (1 km) nowcasts with ML, synoptic (~10 km) for days | Years of station or reanalysis data; spatial consistency needed. | Historical wind; pressure fields; temperature (stability); orography | Ensemble spread; quantile regression【22†L161-L170】 | Reviews show deep learning outperforms traditional methods【22†L154-L163】. E.g., Alves et al. (2023) find LSTM-based models reduce error to ~0.5 m/s. Meteorological ensembles (e.g. ECMWF) are standard for >1-day forecasts. |
| Wind-Chill Temp. | °C | Derived from wind and air temp forecasts | Not forecasted independently | Depends on wind & air temp resolution (hourly) | – (computed on-the-fly) | Predicted wind speed and air temp (from above forecasts) | Propagated from wind/temp uncertainty | Not forecast directly; computed from wind and temperature forecasts. |
| Hooks Load | (n/a) | – (device measurement thresholds) | N/A | – | – | – | – | Not a continuous environmental variable; forecasting not applicable. |
| Hook Alarms | (n/a) | – | N/A | – | – | – | – | As above. |
| Remaining Seconds | s | – | N/A | – | – | – | – | Not a physical parameter (session timeout). No forecast. |
| Conductivity | mS/cm | Physics (salinity proxy); ARIMA; ML (same as salinity) | Similar to salinity forecasts (hours to days) | Hourly; at measured depth or surface | Similar to salinity requirements (years of record) | Historical conductivity or salinity; temperature; flow | Linked to salinity uncertainty | Conductivity closely follows salinity; forecast via salinity models. |
| Salinity | PSU | Hydrodynamic models (salinity transport with tides/flows); ARIMA; ML (SVR, ANN, LSTM, RNN)【36†L1191-L1200】 | Short (days): ML time-series;<br>Medium (weeks): river discharge + tides models; Long (months): seasonal climatology | Daily or hourly (buoy); ocean models ~km resolution | Years of salinity records (buoys/CTDs); critical to capture tidal cycles【36†L1191-L1199】 | Past salinity; river inflow; tidal height; temperature; wind | Ensembles; Bayesian NN | Nicolas et al. (2023) found ANN/SVR/RFR captured seasonal and tidal salinity patterns, with SVR best【36†L1191-L1199】. Similar ML use in estuaries shows skill up to ~30 days. |
| Sound Speed | (derived) | – (computed from T, S, pressure) | N/A | – | – | T,S, pressure | Derived | Computed from water properties (e.g., UNESCO formula). Forecast via T/S predictions. |
| Max Wave Height | m | Wave models (WAM, SWAN); Statistical (Gumbel, Weibull); ML/DL (ANN, LSTM, CNN); hybrid (decomposition+LSTM) | Very short (0–6h): local ML/nowcast;<br>Short (1–3 days): numerical wave model ensemble; <br>Medium (5–10+ days): global wave model | Hours (buoy reports) to a day (synoptic). Phys models give 6h–3d; ML could give per-buoy nowcasts. | Years of buoy wave data (3D, often limited). Uniform data needed. | Historical wave series; wind speed/direction; pressure (for storm surge) | Ensembles of wave model runs; Monte Carlo of input winds | Machine learning (e.g. LSTM+SSA) has greatly improved short-term wave height forecasts【16†L178-L187】. Traditional WAM/SWAN models (with wind input) remain standard for >1-day forecasts. |
| Significant Wave Height | m | See Max Wave Height | Same as Max Wave Height | Same as Max Wave Height | Same as Max Wave Height | Same as Max Wave Height | Same as Max Wave Height | ML models (LSTM/RNN) now routinely exceed older models【18†L278-L287】. E.g., Fu et al. (2022) use hybrid ensembles for multi-scale improvement【18†L303-L312】. |
| Peak Wave Energy Period | s | Wave models; Statistical; ML (LSTM) | Same as wave height | Hours to daily; wave buoys or models output period | Years of buoy data (limited); models use winds | Same as wave height | Ensembles | Forecasts of period typically come with wave models; fewer ML studies focus solely on period. Likely predicted alongside height. |
| Significant Wave Period | s | Same as Peak Period | Same as peak period | Same as other wave periods | Same | Same as period | Same | Often predicted by the same models that predict height; no separate studies found. |
| Zero-Crossing Period | s | Same as other wave periods | Same as wave periods | Same | Same | Same | Same | Same as above; rarely treated separately. |
| Water Level | m | (Synonym of Tide Level; see above) | As tide level | As tide level | As tide level | As tide level | As tide level | Same as "Tide Level" – often term for sea level at mooring. |
| Days Remaining | days | – (non-physical system parameter) | N/A | – | – | – | – | Not an environmental variable; forecasting not applicable. |

Each parameter above is summarized by a concise note: modeling methods, horizon, and data considerations. In general, short-range (minutes–hours) forecasts benefit from high-frequency data and ML nowcasting (e.g. CNN, LSTM on buoy sensors or radar), while medium-range (days) relies on physical ensembles (NWP or ocean models) often combined with ML adjustments. Physical laws (harmonic tides, wave dynamics) set long-term predictability limits, whereas ML can capture local nonlinear patterns for immediate forecasts. In many cases (currents, waves, winds), ensemble forecasts or Bayesian/quantile methods are used for uncertainty (e.g. ensemble Kalman filtering for ocean models, dropout ensembles in neural nets). For non-physical fields (e.g. hook loads, session timers), forecasting is inapplicable.

## Prioritized Data & Literature Sources

- Datasets:
– Tides: NOAA CO‑OPS (Tides & Currents) API【45†L1-L4】 (official tide gauge data, predictions)
– Waves/Buoys: NOAA NDBC (buoy network for wave, wind, pressure data)
– Meteorology: NOAA/NCEP (GFS reanalysis and forecasts), ECMWF (ENS12 reanalysis)
– Ocean: Copernicus Marine Environment Monitoring Service (e.g. GLOBAL_ANALYSIS_FORECAST_PHY_001_024 for currents/temps)
– HF Radar Currents: U.S. IOOS HF Radar (coastal currents)
– Regional Systems: e.g. NOAA SLR for storm surge, USACE ADCIRC model data
– Satellite/Remote: NASA (MODIS SST), ESA (SMOS/SMAP for salinity), Himawari/GOES (cloud/RAD)
– Reanalysis: HYCOM or GLORYS for ocean currents, NOAA OISST for sea temps.

- Key References:
– Huang et al. (2024), Water – VMD+LSTM tidal prediction model【9†L13-L21】.
– Cheng et al. (2024), ArXiv – Wavelet-CNN model for tidal current forecasting【14†L1259-L1268】.
– Farhangi et al. (2023), JMSE – CNN/LSTM for hourly SST prediction【31†L547-L556】.
– Fan et al. (2023), JMSE – Hybrid LSTM+decomposition for significant wave height【18†L278-L287】.
– Farshadnia et al. (2023), Computers – Review of ML in wind nowcasting (LSTM, GRU performance)【22†L154-L163】.
– Nicoletta et al. (2024), Frontiers in Earth Science – ML nowcasting of marine fog (LSTM classification)【43†L43-L46】.
– Copernicus Marine Service – operational ocean forecast models (wave, current reanalysis) (https://marine.copernicus.eu)
– NOAA Joint Hurricane Testbed – applied ML for storm surge (e.g., Hamill et al. 2019).

These sources provide data access and evidence for the effectiveness of various methods in marine forecasting. Together, they support the table’s recommendations for each parameter.